Student Guide: Mastering Network Automation with MCP

A comprehensive guide to understanding and extending the MCP-enabled network automation lab.

Table of Contents

- Introduction: What You're Looking At

- The Big Picture: How Everything Works Together

- Understanding Your Lab Environment

- Deep Dive: The MCP Server Code Explained

- Deep Dive: The LibreChat Configuration Explained

- How to Verify Everything is Working

- Understanding the Data Flow

- What You Can Modify (Next Steps)

- Troubleshooting Common Issues

- Glossary of Terms

Introduction: What You're Looking At

Welcome! You have been given a fully working system that allows you to talk to a Cisco router using natural language through an AI assistant. This is not science fiction—this is the future of network management that major vendors are already building.

What This Lab Demonstrates

Instead of typing complex Cisco commands like show ip interface brief or configure terminal, you can simply ask an AI assistant questions like:

"Show me all the interfaces on the router"

"Configure GigabitEthernet0/1 with IP address 10.1.1.1"

"What is the status of all my network interfaces?"

The AI understands your request, connects to the router, executes the proper commands, and gives you back the results in plain English.

Your Goal in This Lab

Your goals are to:

Understand how the different pieces work together

Explore what each line of code does (explained in plain English)

Verify that your system is functioning correctly

Prepare to add your own custom features in future labs

The Big Picture: How Everything Works Together

The Three Components

Your lab has three machines that work together like a team:

┌──────────────┐         ┌──────────────┐         ┌──────────────┐

│   YOU        │         │   TRANSLATOR │         │   ROUTER     │

│  (Client)    │◄───────►│   (Server)   │◄───────►│  (Device)    │

│ linuxdesktop3│   HTTP  │ linuxdesktop4│   SSH   │ 172.18.2.6   │

│ 172.18.0.2   │   SSE   │ 172.18.1.2   │ Netmiko │              │

│ LibreChat    │         │ FastAPI+MCP  │         │ Cisco IOS    │

└──────────────┘         └──────────────┘         └──────────────┘

Let me explain each component in simple terms:

Component 1: The Client (linuxdesktop3 - Your Machine)

What it is: A chat interface called LibreChat running in Docker containers on your Ubuntu machine.

What it does:

Provides you with a web-based chat interface (like ChatGPT)

Connects to an AI model (Claude from Anthropic) that understands natural language

Sends your requests to the MCP Server when you need router information

Displays the results back to you in a friendly format

Think of it as: The front desk at a hotel—you tell them what you need, and they coordinate with the right people to make it happen.

Component 2: The MCP Server (linuxdesktop4 - The Translator)

What it is: A Python program using the FastAPI web framework that implements the Model Context Protocol (MCP).

What it does:

Acts as a bridge between the AI assistant and your Cisco router

Translates AI requests into actual router commands

Executes those commands on the router using SSH

Sends the router's response back to the AI

Think of it as: A translator who speaks both "AI language" and "Cisco router language." When the AI says "get interfaces," the translator knows to run show ip interface brief on the router.

Component 3: The Router (The Target Device)

What it is: A real Cisco router running IOS (the Cisco operating system).

What it does:

Responds to SSH connections

Executes commands sent to it

Returns configuration and status information

Think of it as: The actual device you're managing—like a thermostat in a smart home system. You don't talk to it directly; you talk to your phone, which talks to the thermostat.

Understanding Your Lab Environment

Network Configuration

All three components are connected on the same network so they can communicate with each other:

Software Installed

Your lab environment has these key software packages pre-installed:

On MCP Client (linuxdesktop3):

Docker & Docker Compose: Runs LibreChat in isolated containers

Node.js 20.x: JavaScript runtime required by LibreChat

LibreChat: The AI chat interface

On MCP Server (linuxdesktop4):

Python 3.11: Programming language for the MCP server

FastAPI: Web framework for building APIs quickly

Uvicorn: Web server that runs the FastAPI application

MCP SDK: Python library for Model Context Protocol

Netmiko: Python library for connecting to network devices via SSH

Don't worry if you don't know what all these terms mean yet. Think of them like ingredients in a recipe—you don't need to understand chemistry to follow a recipe, but knowing what each ingredient does helps you cook better.

Deep Dive: The MCP Server Code Explained

Now let's look at the actual code running on linuxdesktop4 (the MCP Server). This code is located at:

/home/ubuntu/cisco_mcp/app/main.py

Section 1: Importing Libraries (Lines 1-6)

from fastapi import FastAPI, Request

from fastapi.responses import StreamingResponse, Response

from mcp.server import Server

from mcp.server.sse import SseServerTransport

from mcp.types import Tool, TextContent

from netmiko import ConnectHandler

What this means in plain English:

These lines are like telling Python "I need to use these toolkits." It's similar to opening your toolbox before starting a repair job.

from fastapi import FastAPI, Request: Brings in the FastAPI web framework so we can create a web server that listens for requests

from fastapi.responses import StreamingResponse, Response: Imports tools for sending data back to whoever asked for it

from mcp.server import Server: Brings in the main MCP server component from the Model Context Protocol library

from mcp.server.sse import SseServerTransport: Imports the SSE (Server-Sent Events) transport layer, which is how the server communicates with LibreChat in real-time

from mcp.types import Tool, TextContent: Imports the data structures for defining tools and returning text responses

from netmiko import ConnectHandler: Brings in Netmiko, which is a Python library that knows how to connect to network devices like Cisco routers using SSH

Why we need these: Without importing these libraries, Python wouldn't know how to create a web server, connect to routers, or use the MCP protocol.

Section 2: Router Configuration (Lines 8-14)

ROUTER_CONFIG = {

'device_type': 'cisco_ios',

'host': '172.18.2.6',

'username': 'admin',

'password': 'admin',

'secret': 'admin',

}

What this means in plain English:

This creates a dictionary (think of it as a contact card) with all the information needed to connect to the Cisco router.

ROUTER_CONFIG: This is the name we're giving to this set of information. It's written in ALL_CAPS because it's a constant—something that won't change while the program runs

'device_type': 'cisco_ios': Tells Netmiko "you're connecting to a Cisco router running IOS"

'host': '172.18.2.6': This is the IP address of the router (like a phone number)

'username': 'admin': The username to log into the router

'password': 'admin': The password to log into the router

'secret': 'admin': The "enable" password (needed to run configuration commands on Cisco devices)

Why we need this: Every time the MCP server needs to connect to the router, it uses this information. Instead of typing it repeatedly, we save it once here.

Security Note: In a production environment, you would never hardcode passwords like this. They would be stored securely in environment variables or a secrets manager.

Section 3: Initializing the Application (Lines 16-18)

app = FastAPI(title="Cisco Router MCP Server")

mcp_server = Server("cisco-router-mcp")

sse_transport = SseServerTransport("/messages/")

What this means in plain English:

These three lines set up the foundation of our application—like laying the groundwork before building a house.

app = FastAPI(title="Cisco Router MCP Server"): Creates a new FastAPI web application with the title "Cisco Router MCP Server." This is the web server that will listen for incoming requests

mcp_server = Server("cisco-router-mcp"): Creates an MCP server instance with the name "cisco-router-mcp." This is the actual MCP protocol handler that manages tools and communication

sse_transport = SseServerTransport("/messages/"): Creates the SSE (Server-Sent Events) transport layer, which handles real-time, bidirectional communication between LibreChat and the MCP server. The "/messages/" part tells it where to listen for incoming messages

Why we need these three things:

app is the web server (the front door)

mcp_server is the brain (handles MCP protocol logic)

sse_transport is the communication system (manages the conversation with LibreChat)

All three work together to make the system function.

Section 4: Defining Available Tools (Lines 20-42)

@mcp_server.list_tools()

async def list_tools() -> list[Tool]:

return [

Tool(

name="get_interfaces",

description="Get all router interfaces and their status",

inputSchema={"type": "object", "properties": {}}

),

Tool(

name="configure_interface",

description="Configure an interface with IP address",

inputSchema={

"type": "object",

"properties": {

"interface": {"type": "string"},

"ip_address": {"type": "string"}

},

"required": ["interface", "ip_address"]

}

),

]

What this means in plain English:

This section is like creating a menu at a restaurant—it lists all the things the AI can ask the MCP server to do.

Breaking it down line by line:

@mcp_server.list_tools(): This is called a "decorator" in Python. Think of it as a label that says "this function is special—it defines what tools are available to the AI"

async def list_tools() -> list[Tool]:: This creates a function (a reusable block of code) named list_tools. The async means it can handle multiple requests at once without blocking. The -> list[Tool] tells Python "this function returns a list of Tool objects"

return [: Start returning a list of tools

Tool #1: get_interfaces

Tool(

name="get_interfaces",

description="Get all router interfaces and their status",

inputSchema={"type": "object", "properties": {}}

)

This defines the first tool. Let's break it down:

name="get_interfaces": The name of this tool. When the AI wants to use it, it will refer to it by this name

description="Get all router interfaces and their status": A human-readable explanation of what this tool does. The AI reads this to decide when to use the tool

inputSchema={"type": "object", "properties": {}}: This describes what inputs the tool needs. In this case, the empty properties: {} means it doesn't need any inputs—it just runs and returns interface information

Tool #2: configure_interface

Tool(

name="configure_interface",

description="Configure an interface with IP address",

inputSchema={

"type": "object",

"properties": {

"interface": {"type": "string"},

"ip_address": {"type": "string"}

},

"required": ["interface", "ip_address"]

}

)

This defines the second tool:

name="configure_interface": The name of this tool

description="Configure an interface with IP address": Explains what it does

inputSchema: This tool requires inputs:

"interface": {"type": "string"}: The AI must provide the interface name (like "GigabitEthernet0/1") as text

"ip_address": {"type": "string"}: The AI must provide an IP address as text

"required": ["interface", "ip_address"]: Both inputs are mandatory

Why this matters: When you ask LibreChat "Configure GigabitEthernet0/1 with IP 10.1.1.1," the AI (Claude) reads this tool definition and knows:

There's a tool called configure_interface that can do this

It needs to provide two pieces of information: the interface name and the IP address

It then calls the tool with those arguments

Section 5: Implementing Tool Logic (Lines 44-71)

@mcp_server.call_tool()

async def call_tool(name: str, arguments: dict) -> list[TextContent]:

if name == "get_interfaces":

try:

connection = ConnectHandler(**ROUTER_CONFIG)

output = connection.send_command("show ip interface brief")

connection.disconnect()

return [TextContent(type="text", text=f"Router1 Interfaces:\n\n{output}")]

except Exception as e:

return [TextContent(type="text", text=f"Error: {str(e)}")]

elif name == "configure_interface":

interface = arguments.get("interface")

ip_address = arguments.get("ip_address")

try:

connection = ConnectHandler(**ROUTER_CONFIG)

connection.enable()

commands = [f"interface {interface}", f"ip address {ip_address} 255.255.255.0", "no shutdown"]

output = connection.send_config_set(commands)

connection.disconnect()

return [TextContent(type="text", text=f"Configured {interface} with {ip_address}\n\n{output}")]

except Exception as e:

return [TextContent(type="text", text=f"Error: {str(e)}")]

return [TextContent(type="text", text=f"Unknown tool: {name}")]

What this means in plain English:

This is the actual implementation of the tools—the code that runs when the AI calls one of the tools we defined above.

Breaking it down:

@mcp_server.call_tool(): Another decorator that says "this function handles tool execution"

async def call_tool(name: str, arguments: dict) -> list[TextContent]:: Creates a function that:

Takes two inputs: name (which tool to run) and arguments (the inputs for that tool)

Returns a list of TextContent objects (the results)

Handling Tool #1: get_interfaces

if name == "get_interfaces":

try:

connection = ConnectHandler(**ROUTER_CONFIG)

output = connection.send_command("show ip interface brief")

connection.disconnect()

return [TextContent(type="text", text=f"Router1 Interfaces:\n\n{output}")]

except Exception as e:

return [TextContent(type="text", text=f"Error: {str(e)}")]

Let's break this down line by line:

if name == "get_interfaces":: If the AI is calling the get_interfaces tool...

try:: Python's way of saying "try to do the following, but if something goes wrong, handle it gracefully"

connection = ConnectHandler(**ROUTER_CONFIG): Connect to the router using SSH with the credentials we defined earlier. This is like dialing into the router

output = connection.send_command("show ip interface brief"): Send the Cisco command show ip interface brief to the router and store the result in the variable output

connection.disconnect(): Hang up the SSH connection (like hanging up a phone call)

return [TextContent(type="text", text=f"Router1 Interfaces:\n\n{output}")]: Send the router's response back to the AI in a format it understands

except Exception as e:: If anything went wrong (network error, wrong password, etc.)...

return [TextContent(type="text", text=f"Error: {str(e)}")]: Return an error message instead of crashing

Handling Tool #2: configure_interface

elif name == "configure_interface":

interface = arguments.get("interface")

ip_address = arguments.get("ip_address")

try:

connection = ConnectHandler(**ROUTER_CONFIG)

connection.enable()

commands = [f"interface {interface}", f"ip address {ip_address} 255.255.255.0", "no shutdown"]

output = connection.send_config_set(commands)

connection.disconnect()

return [TextContent(type="text", text=f"Configured {interface} with {ip_address}\n\n{output}")]

except Exception as e:

return [TextContent(type="text", text=f"Error: {str(e)}")]

Breaking this down:

elif name == "configure_interface":: "else if" the tool being called is configure_interface...

interface = arguments.get("interface"): Extract the interface name from the arguments the AI provided

ip_address = arguments.get("ip_address"): Extract the IP address from the arguments

connection = ConnectHandler(**ROUTER_CONFIG): Connect to the router via SSH

connection.enable(): Enter "enable mode" on the Cisco router (like becoming an admin on a computer)

commands = [...]: Create a list of Cisco IOS commands to run:

interface {interface}: Select the interface (e.g., "interface GigabitEthernet0/1")

ip address {ip_address} 255.255.255.0: Assign the IP address with a /24 subnet mask

no shutdown: Enable the interface (Cisco interfaces are "shutdown" by default)

output = connection.send_config_set(commands): Send all three commands to the router at once

connection.disconnect(): Close the SSH connection

return [TextContent(...)]: Send back a success message with the router's response

Fallback for Unknown Tools:

return [TextContent(type="text", text=f"Unknown tool: {name}")]

If the AI somehow calls a tool that doesn't exist, return an error message instead of crashing.

Section 6: SSE Endpoint (Lines 73-76)

@app.get("/sse")

async def sse_endpoint(request: Request):

async with sse_transport.connect_sse(request.scope, request.receive, request._send) as streams:

await mcp_server.run(streams[^0], streams[^1], mcp_server.create_initialization_options())

What this means in plain English:

This creates the main connection point where LibreChat connects to the MCP server.

Breaking it down:

@app.get("/sse"): This is a FastAPI decorator that says "when someone makes a GET request to http://172.18.1.2:8000/sse, run this function"

async def sse_endpoint(request: Request):: Defines the function that handles SSE connections

async with sse_transport.connect_sse(...) as streams:: Establishes a Server-Sent Events connection. Think of this as opening a two-way radio channel between LibreChat and the MCP server

await mcp_server.run(streams, streams[^19], ...): Starts the MCP server and keeps it running, processing requests and sending responses through the SSE connection[1]

Why this is important: This is the "always-on" connection that allows LibreChat to communicate with your MCP server in real-time. When you type a message in LibreChat, it travels through this connection.

Section 7: Messages Endpoint (Lines 78-80)

@app.post("/messages/")

async def messages_endpoint(request: Request):

return await sse_transport.handle_post_message(request.scope, request.receive, request._send)

What this means in plain English:

This creates a second endpoint that handles incoming messages from LibreChat.

Breaking it down:

@app.post("/messages/"): When someone makes a POST request to http://172.18.1.2:8000/messages/, run this function

async def messages_endpoint(request: Request):: Defines the function

return await sse_transport.handle_post_message(...): Hands off the incoming message to the SSE transport layer to process

Why both endpoints are needed: SSE (Server-Sent Events) is a one-way protocol (server → client). To make it bidirectional:

/sse (GET) is used for the server to send events to the client

/messages/ (POST) is used for the client to send messages to the server

Together, they create a full two-way conversation.

Section 8: Root Endpoint (Lines 82-84)

@app.get("/")

def root():

return {"status": "Cisco Router MCP Server Running", "router": "Router1 (172.18.2.6)", "mcp_endpoint": "/sse"}

What this means in plain English:

This is a simple "health check" endpoint that confirms the server is running.

Breaking it down:

@app.get("/"): When someone visits http://172.18.1.2:8000/ in a browser or with curl, run this function

def root():: Note this is not async def—it's a simple synchronous function because it doesn't need to wait for anything

return {...}: Returns a JSON object with status information

Why this is useful: You can test if the MCP server is running by simply visiting http://172.18.1.2:8000/ in a browser. You should see:

{
  "status": "Cisco Router MCP Server Running",
  "router": "Router1 (172.18.2.6)",
  "mcp_endpoint": "/sse"
}

This is your "quick health check" to verify the server is alive.

Deep Dive: The LibreChat Configuration Explained

Now let's look at the configuration on linuxdesktop3 (the MCP Client). The key configuration file is:

/home/ubuntu/LibreChat/librechat.yaml

Here's the content:

version: 1.2.8
cache: true
endpoints:
  custom:
    - name: "OpenRouter"
      apiKey: "${OPENROUTER_KEY}"
      baseURL: "https://openrouter.ai/api/v1"
      models:
        default: ["meta-llama/llama-3-70b-instruct"]
        fetch: true
      titleConvo: true
      titleModel: "meta-llama/llama-3-70b-instruct"
      dropParams: ["stop"]
      modelDisplayLabel: "OpenRouter"
mcpServers:
  cisco-router:
    name: "Cisco Router Manager"
    type: sse
    url: "http://172.18.1.2:8000/sse"
    chatMenu: true
    serverInstructions: |
      You are managing Cisco Router1 (172.18.2.6) via MCP server.
      Available tools:
      - get_interfaces: Show all router interfaces
      - configure_interface: Configure interface with IP address
      Always confirm before making configuration changes.

Let's break this down section by section:

Section 1: Basic Configuration

version: 1.2.8
cache: true

What this means:

version: 1.2.8: The version of the LibreChat configuration format being used. This ensures compatibility

cache: true: Enables caching of responses to make the system faster and reduce API costs

Section 2: LLM Endpoint Configuration

endpoints:
  custom:
    - name: "OpenRouter"
      apiKey: "${OPENROUTER_KEY}"
      baseURL: "https://openrouter.ai/api/v1"
      models:
        default: ["meta-llama/llama-3-70b-instruct"]
        fetch: true
      titleConvo: true
      titleModel: "meta-llama/llama-3-70b-instruct"
      dropParams: ["stop"]
      modelDisplayLabel: "OpenRouter"

What this means in plain English:

This section configures which AI model LibreChat will use to understand your natural language requests.

endpoints: custom:: Defines custom AI endpoints (as opposed to built-in ones like OpenAI)

name: "OpenRouter": A friendly name for this endpoint

apiKey: "${OPENROUTER_KEY}": The API key to authenticate with OpenRouter. The ${...} syntax means "read this value from the environment variables in the .env file"[1]

baseURL: "https://openrouter.ai/api/v1": The web address where OpenRouter's API is located

models: default: ["meta-llama/llama-3-70b-instruct"]: Use Meta's Llama 3 70B model as the default AI. This is a powerful open-source language model

fetch: true: Automatically fetch the list of available models from OpenRouter

titleConvo: true: Automatically generate conversation titles based on content

titleModel: Use the same Llama model for generating titles

dropParams: ["stop"]: Remove certain parameters that might cause compatibility issues

modelDisplayLabel: "OpenRouter": How the model appears in the UI

Why this matters: LibreChat needs an AI model (the "brain") to understand your questions and decide when to use MCP tools. In this case, it's using Meta's Llama 3 via OpenRouter.

Section 3: MCP Server Configuration (The Important Part!)

mcpServers:
  cisco-router:
    name: "Cisco Router Manager"
    type: sse
    url: "http://172.18.1.2:8000/sse"
    chatMenu: true
    serverInstructions: |
      You are managing Cisco Router1 (172.18.2.6) via MCP server.
      Available tools:
      - get_interfaces: Show all router interfaces
      - configure_interface: Configure interface with IP address
      Always confirm before making configuration changes.

What this means in plain English:

This is where LibreChat learns about your MCP server and how to connect to it.

Breaking it down:

mcpServers:: Start of MCP server definitions

cisco-router:: Internal identifier for this MCP server (you can have multiple MCP servers)

name: "Cisco Router Manager": The friendly name that appears in LibreChat's interface

type: sse: The connection type is SSE (Server-Sent Events). This matches the /sse endpoint we defined in the MCP server code[3][1]

url: "http://172.18.1.2:8000/sse": The full URL where LibreChat should connect to reach the MCP server. Notice:

172.18.1.2: The MCP server's IP address

:8000: The port the FastAPI server is listening on

/sse: The SSE endpoint we defined earlier

chatMenu: true: Show this MCP server as an option in LibreChat's chat menu (so you can toggle it on/off)

serverInstructions:: These are instructions given to the AI to help it understand how to use the MCP server properly. Let's break down these instructions:

You are managing Cisco Router1 (172.18.2.6) via MCP server.: Tells the AI the context—it's managing a specific router

Available tools:: Lists what the AI can do

get_interfaces: Shows all router interfaces

configure_interface: Configures an interface with an IP address

Always confirm before making configuration changes.: Instructs the AI to ask for user confirmation before running configuration commands (a safety measure!)

Why this is critical: This configuration is what connects LibreChat to your MCP server. Without it, LibreChat wouldn't know the MCP server exists or how to reach it.

Environment Variables (.env file)

In addition to librechat.yaml, there's a .env file that contains sensitive information:

OPENROUTER_KEY=your-api-key-here
PORT=3080
UID=1000
GID=1000

What these mean:

OPENROUTER_KEY: Your API key for accessing OpenRouter (or you might use ANTHROPIC_API_KEY for Claude directly)

PORT=3080: The port LibreChat runs on

UID and GID: User and group IDs for Docker containers (Linux permission management)

Why separate files: Sensitive data like API keys should never be committed to version control. Keeping them in .env (which is ignored by git) is a security best practice.

How to Verify Everything is Working

Let's walk through verification steps to ensure your lab environment is functioning correctly.

Step 1: Verify Network Connectivity

From linuxdesktop3 (MCP Client), test that you can reach the MCP server:

ping -c 3 172.18.1.2

Expected output:

PING 172.18.1.2 (172.18.1.2) 56(84) bytes of data.
64 bytes from 172.18.1.2: icmp_seq=1 ttl=64 time=0.345 ms
64 bytes from 172.18.1.2: icmp_seq=2 ttl=64 time=0.298 ms
64 bytes from 172.18.1.2: icmp_seq=3 ttl=64 time=0.312 ms

What this confirms: The MCP Client can reach the MCP Server over the network.

From linuxdesktop4 (MCP Server), test that you can reach the router:

ping -c 3 172.18.2.6

Expected output: Similar ping responses from the router.

What this confirms: The MCP Server can reach the Cisco router.

Step 2: Verify the MCP Server is Running

On linuxdesktop4, check if the MCP server process is running:

cd ~/cisco_mcp

ps aux | grep uvicorn

Expected output: Should show a running uvicorn process.

If it's not running, start it:

cd ~/cisco_mcp

./run.sh

Expected output:

Starting Cisco MCP Server on http://0.0.0.0:8000...
INFO:     Started server process [^12345]
INFO:     Waiting for application startup.
INFO:     Application startup complete.
INFO:     Uvicorn running onhttp://0.0.0.0:8000 (Press CTRL+C to quit)

What this means: The FastAPI server is running and listening for connections.[1]

Step 3: Test the MCP Server Health Endpoint

From linuxdesktop3, test the MCP server's root endpoint:

curl http://172.18.1.2:8000/

Expected output:

{"status":"Cisco Router MCP Server Running","router":"Router1 (172.18.2.6)","mcp_endpoint":"/sse"}

What this confirms: The MCP server is responding to HTTP requests correctly.

Step 4: Verify LibreChat is Running

On linuxdesktop3, check if LibreChat containers are running:

cd ~/LibreChat

docker-compose ps

Expected output:

NAME                IMAGE                                              STATUS

LibreChat           ghcr.io/danny-avila/librechat-dev:latest          Up

chat-mongodb        mongo                                              Up

chat-meilisearch    getmeili/meilisearch:v1.12.3                      Up

rag_api             ghcr.io/danny-avila/librechat-rag-api-dev-lite   Up

vectordb            pgvector/pgvector:0.8.0-pg15-trixie               Up

What this confirms: All LibreChat services are running.

If containers are not running, start them:

cd ~/LibreChat

sudo docker-compose up -d

Step 5: Access LibreChat in Your Browser

Open a web browser and navigate to:

http://172.18.0.2:3080

or if you're on the same machine:

http://localhost:3080

Expected result: You should see the LibreChat login/registration page.

Step 6: Verify MCP Connection in LibreChat

Log in or create an account in LibreChat

Look for the "Cisco Router Manager" option in the interface (this comes from your librechat.yaml configuration)

Enable the MCP server by selecting it in the chat menu

Type a test message:

Show me the router interfaces

Observe the AI's response

Expected behavior:

The AI (Llama 3) recognizes you're asking for router interface information

It calls the get_interfaces tool on the MCP server

The MCP server connects to the router via SSH

The router returns interface information

The MCP server sends it back to LibreChat

The AI formats the response in natural language and displays it to you

Example response:

Here are the interfaces on Router1:

Interface                  IP-Address      OK? Method Status                Protocol
GigabitEthernet0/0         192.168.1.1     YES NVRAM  up                    up
GigabitEthernet0/1         unassigned      YES NVRAM  administratively down down
GigabitEthernet0/2         10.0.0.1        YES NVRAM  up                    up
Loopback0                  1.1.1.1         YES NVRAM  up                    up

All interfaces are functioning properly.

Step 7: Check Logs for Debugging

If something doesn't work, check the logs:

LibreChat logs:

cd ~/LibreChat

docker compose logs -f api | grep -i mcp

MCP Server logs:

The MCP server displays logs in the terminal where you ran ./run.sh. Look for messages like:

INFO:     172.18.0.2:52341 - "GET /sse HTTP/1.1" 200 OK

This shows LibreChat successfully connected to the SSE endpoint.

Understanding the Data Flow

Let's trace exactly what happens when you ask LibreChat "Show me router interfaces":

Step-by-Step Message Flow

┌─────────────────────────────────────────────────────────────────────┐

│ Step 1: User types "Show me router interfaces" in LibreChat        │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 2: LibreChat sends the message to Meta Llama 3 (via OpenRouter)│

│         along with the list of available MCP tools                  │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 3: Llama 3 analyzes the request and decides:                  │

│         "This requires the 'get_interfaces' tool"                  │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 4: LibreChat sends a tool call request to MCP server:         │

│         POST to http://172.18.1.2:8000/messages/                   │

│         {                                                           │

│           "tool": "get_interfaces",                                │

│           "arguments": {}                                          │

│         }                                                           │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 5: MCP server receives the request and runs call_tool()       │

│         function, which executes:                                   │

│         - connection = ConnectHandler(**ROUTER_CONFIG)             │

│         - output = connection.send_command("show ip interface brief")│

│         - connection.disconnect()                                   │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 6: MCP server connects to Router1 (172.18.2.6) via SSH       │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 7: Router1 executes "show ip interface brief" and returns:    │

│         Interface              IP-Address      OK? Method Status... │

│         GigabitEthernet0/0     192.168.1.1     YES NVRAM  up...    │

│         ...                                                         │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 8: MCP server sends the router output back to LibreChat:      │

│         {                                                           │

│           "type": "text",                                          │

│           "text": "Router1 Interfaces:\n\n[output]"               │

│         }                                                           │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 9: LibreChat forwards the tool result to Llama 3              │

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 10: Llama 3 formats the raw output into natural language:     │

│          "Here are the interfaces on Router1: [formatted response]"│

└─────────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────────┐

│ Step 11: LibreChat displays the final response to the user         │

└─────────────────────────────────────────────────────────────────────┘

Total time: Typically 2-5 seconds from your question to the final answer.

What You Can Modify (Next Steps)

Now that you understand how everything works, here are areas you can safely modify to learn more:

Adding a New Tool

The most common modification is adding a new tool. For example, let's say you want to add a tool to view the router's running configuration.

Step 1: Add the tool definition in the list_tools() function:

Tool(

name="get_running_config",

description="Get the running configuration of the router",

inputSchema={"type": "object", "properties": {}}

)

Step 2: Add the implementation in the call_tool() function:

elif name == "get_running_config":

try:

connection = ConnectHandler(**ROUTER_CONFIG)

output = connection.send_command("show running-config")

connection.disconnect()

return [TextContent(type="text", text=f"Running Config:\n\n{output}")]

except Exception as e:

return [TextContent(type="text", text=f"Error: {str(e)}")]

Step 3: Restart the MCP server:

Press CTRL+C to stop the current server

./run.sh

Step 4: Test it in LibreChat:

Show me the router's running configuration

The AI should now use your new tool!

Modifying Tool Parameters

You can change the inputSchema to require different inputs. For example, to add a description parameter to configure_interface:

Tool(

name="configure_interface",

description="Configure an interface with IP address and description",

inputSchema={

"type": "object",

"properties": {

"interface": {"type": "string"},

"ip_address": {"type": "string"},

"description": {"type": "string"}

},

"required": ["interface", "ip_address", "description"]

}

)

Then update the implementation:

elif name == "configure_interface":

interface = arguments.get("interface")

ip_address = arguments.get("ip_address")

description = arguments.get("description")

try:

connection = ConnectHandler(**ROUTER_CONFIG)

connection.enable()

commands = [

f"interface {interface}",

f"description {description}",

f"ip address {ip_address} 255.255.255.0",

"no shutdown"

]

output = connection.send_config_set(commands)

connection.disconnect()

return [TextContent(type="text", text=f"Configured {interface} with {ip_address}\n\n{output}")]

except Exception as e:

return [TextContent(type="text", text=f"Error: {str(e)}")]

Now when you ask to configure an interface, the AI will also ask for a description!

Changing the Router Connection

If you need to manage a different router, simply update the ROUTER_CONFIG dictionary:

ROUTER_CONFIG = {

'device_type': 'cisco_ios',

'host': '172.18.2.7',  # Changed IP

'username': 'admin',

'password': 'newpassword',  # Changed password

'secret': 'admin',

}

Remember: Restart the MCP server after making changes.

Updating LibreChat Configuration

You can modify librechat.yaml to change how the AI behaves:

serverInstructions: |
  You are managing Cisco Router1 (172.18.2.6) via MCP server.
  Available tools:

- get_interfaces: Show all router interfaces

- configure_interface: Configure interface with IP address

- get_running_config: Display the running configuration

Always confirm before making configuration changes.
  When showing configurations, summarize the key points for readability.

After modifying librechat.yaml, restart LibreChat:

cd ~/LibreChat

docker compose restart api

The new instructions will guide the AI's behavior.

Glossary of Terms

Here are definitions of all the technical terms used in this guide:

API (Application Programming Interface): A way for different software programs to talk to each other. Like a waiter taking your order to the kitchen.

Async (Asynchronous): Code that can handle multiple tasks at the same time without waiting for each to finish. Like a chef cooking multiple dishes simultaneously.

Claude: An AI language model created by Anthropic, similar to ChatGPT. It's the "brain" that understands your natural language requests.

Decorator (Python): The @ symbol followed by a function name (like @app.get()). It modifies how the function below it behaves—like adding a label to a file folder.

Dictionary (Python): A data structure that stores key-value pairs. Like a real dictionary where you look up a word (key) to find its meaning (value).

Docker: Software that runs applications in isolated containers, ensuring they have everything they need to run consistently.

Docker Compose: A tool for defining and running multi-container Docker applications using a YAML file.

FastAPI: A modern Python web framework for building APIs quickly. It handles HTTP requests and responses.

Function: A reusable block of code that performs a specific task. Like a recipe you can follow repeatedly.

HTTP (HyperText Transfer Protocol): The language web browsers and servers use to communicate. When you visit a website, your browser makes HTTP requests.

Import (Python): Loading external code libraries into your program. Like borrowing tools from a toolbox.

IP Address: A unique number that identifies a device on a network. Like a phone number for computers.

JSON (JavaScript Object Notation): A format for structuring data that both humans and computers can read. Looks like: {"name": "value"}.

JSON-RPC: A protocol for calling functions on remote servers using JSON. MCP uses JSON-RPC 2.0 for communication.

LibreChat: An open-source AI chat interface that supports multiple AI models and MCP servers.

Llama 3: A large language model created by Meta (Facebook). It's an open-source alternative to GPT-4.

MCP (Model Context Protocol): A standardized way for AI applications to connect to external tools and data sources. Think of it as a universal translator between AI and the real world.

MCP Client: The component inside LibreChat that connects to MCP servers and translates between the AI and MCP protocol.

MCP Server: A program that exposes tools and data to AI applications using the MCP protocol.

Netmiko: A Python library specifically designed for connecting to network devices (routers, switches) via SSH.

OpenRouter: A service that provides access to multiple AI models through a single API.

Port: A number that identifies a specific service on a computer. Like apartment numbers in a building (IP address = building, port = apartment).

POST/GET: Types of HTTP requests. GET retrieves data (like reading), POST sends data (like writing).

Python: A popular programming language known for being readable and beginner-friendly.

SSH (Secure Shell): A protocol for securely connecting to and controlling remote computers or devices.

SSE (Server-Sent Events): A technology that allows servers to push updates to web browsers in real-time. Like a news ticker that automatically updates.

Tool (in MCP context): A function that AI can call to perform actions or retrieve information. Like giving the AI a Swiss Army knife.

Transport (in MCP context): The communication method between MCP clients and servers (like SSE or stdio).

Try/Except: Python's way of handling errors gracefully. "Try this code, but if something goes wrong (except), do this instead".

Uvicorn: A web server that runs FastAPI applications. It listens for incoming requests and routes them to your code.

Variable: A container for storing data that can change. Like a labeled box where you put things.

YAML: A human-readable data format used for configuration files. Simpler than JSON.

Congratulations!

This lab demonstrates the future of network automation—where network engineers can manage infrastructure using natural language instead of memorizing complex command syntax.

Have fun exploring the future of network automation!

Resources:

https://modelcontextprotocol.io/docs/getting-started/intro